Week-2

- a) Create and attach an EBS volume to an EC2 instance, and

- scale the instance by increasing CPU and RAM by changing the instance type.

2.Attach and permanently mount an EBS volume to a Linux EC2 instance to ensure data persistence across reboots.

3. Create a snapshot of the attached EBS volume and use it to create and attach a new volume to an EC2 instance in another AWS region.

- a) Create and attach an EBS volume to an EC2 instance, and

- scale the instance by increasing CPU and RAM by changing the instance type.

- Creating  and attaching  an EBS volume to an EC2 instance

Step 1: Create a New EBS Volume

Open the EC2 Dashboard.

From the left-hand menu, select Volumes under Elastic Block Store (EBS).

Click Create Volume.

Choose:

Volume type (gp3/gp2)

Size of the volume

Availability Zone (must be the same as the EC2 instance)

Click Create Volume.

Step 2: Attach the EBS Volume to the Existing EC2 Instance

In the Volumes section, select the newly created volume.

Click Actions → Attach Volume.

Select the target EC2 instance from the list.

Specify the device name (example: /dev/xvdf).

Click Attach.

Step 3: Verify the Volume on the EC2 Instance

Connect to the EC2 instance using SSH.

Check the attached disks:

lsblk

Identify the new volume (e.g., /dev/xvdf).

Note: Ensure that the EBS volume and the EC2 instance belong to the same Availability Zone, as cross-AZ attachment is not supported.

b)Procedure: increasing/decreasing  of CPU and RAM

Step 1.  - Navigate to the EC2 dashboard.

Step 2. - Stop the EC2 instance associated with the EBS volume.

Step 3.  - Click "Actions" and then—instance settings—change the instance type"Modify Volume".

Step 4. -Increase/ decrease the size or change the volume type to a higher performance specification.

Step 5. - Click "Modify" to apply the changes.

Step 6:  Now the start the machine you see with increase /decrease of size and performance

2.Attach and permanently mount an EBS volume to a Linux EC2 instance to ensure data persistence across reboots.

Step 1: Create an EBS Volume

- Log in to AWS Management Console

- Go to EC2 → Volumes

- Click Create Volume

- Choose:

  - Volume Type: gp3 (recommended)

  - Size: (e.g., 20 GB)

  - Availability Zone: Same AZ as EC2 instance

- Click Create Volume

Step 2: Attach the EBS Volume to EC2

- Select the newly created EBS volume

- Click Actions → Attach Volume

- Choose:

  - Instance: Select your Linux EC2 instance

  - Device name: /dev/xvdf

- Click Attach

Step 3: Connect to the EC2 Instance

Connect using SSH:

ssh -i key.pem ec2-user@<public-ip>

Step 4: Verify the Attached Disk

Check all available disks:

lsblk

You will see:

- Root volume: /dev/xvda

- New volume: /dev/xvdf (no partition yet)

Check filesystem:

lsblk -fs

Step 5: Create a Partition

Create a partition on the new volume:

fdisk /dev/xvdf

Inside fdisk:

- Press m → Help menu

- Press n → New partition

- Press p → Primary partition

- Press Enter → Default partition number

- Press Enter → Default first sector

- Press Enter → Default last sector (use full disk)

- Press w → Write changes and exit

Notify kernel:

partprobe

Step 6: Verify the New Partition

lsblk -fs

You should now see:

/dev/xvdf1

Step 7: Format the Partition

Format with XFS filesystem:

mkfs.xfs /dev/xvdf1

Verify:

lsblk -fs

Step 8: Create a Mount Directory

mkdir /mnt/archana

Step 9: Mount the Volume

mount /dev/xvdf1 /mnt/archana

Verify:

df -h

Step 10: Make the Mount Persistent

Get the UUID of the volume:

blkid /dev/xvdf1

Edit fstab:

nano /etc/fstab

Add this line at the end:

UUID=<UUID-value>   /mnt/archana   xfs   defaults,nofail   0   0

Save and exit.

Test fstab:

mount -a

Step 11: Verify Persistence

- Reboot the instance:

reboot

- Reconnect and check:

df -h

➡️ Volume should be mounted automatically.

Step 12: Detach and Reattach Volume (Data Recovery Scenario)

Detach:

- EC2 → Volumes → Select volume

- Actions → Detach volume

Attach to another EC2 instance:

- Attach to new instance (same AZ)

- Login to new instance

- Create mount directory:

mkdir /mnt/archana

- Mount the volume:

mount /dev/xvdf1 /mnt/archana

All previous data will be available

Summary Flow

Create Volume → Attach → lsblk → fdisk → mkfs → mkdir → mount → fstab → reboot

3. Create a snapshot of the attached EBS volume and use it to create and attach a new volume to an EC2 instance in another AWS region.

Attaching Volumes across regions of EC2 using the snapshot

To create a snapshot of an EBS volume and attach it to an instance in another region in AWS, you'll need to follow these steps:

Step 1. Create a Snapshot:

a. Sign in to the AWS Management Console.

b. Navigate to the EC2 Dashboard.

c. Click on "Volumes" in the left-hand navigation pane.

d. Select the EBS volume you want to create a snapshot of.

e. Click on the "Actions" dropdown menu above the volume list and select "Create Snapshot."

f. Provide a name and description for the snapshot.

g. Click on the "Create Snapshot" button to initiate the snapshot creation process.

Step 2. Copy the Snapshot to Another Region:

a. Once the snapshot is created, go to the "Snapshots" section in the EC2 Dashboard.

b. Select the snapshot you just created.

c. Click on the "Actions" dropdown menu above the snapshot list and select "Copy Snapshot."

d. Choose the destination region where you want to copy the snapshot.

e. Click on the "Copy Snapshot" button to initiate the copy process. This may take some time depending on the size of the snapshot and the network speed.

Step 3. Monitor the Snapshot Copy Progress:

a. You can monitor the progress of the snapshot copy by navigating to the "Snapshots" section in the EC2 Dashboard of the source region.

b. Look for the snapshot you copied and check its status. It will change from "pending" to "completed" once the copy process is finished.

Step 4. Switch to the Destination Region:

- Use the region selector in the top-right corner of the AWS Management Console to switch to the destination region where you copied the snapshot.

Step 5. Create a Volume from the Snapshot:

a. In the EC2 Dashboard of the destination region, go to the "Snapshots" section.

b. Find the snapshot you copied from the source region.

c. Click on the snapshot, then click on the "Actions" dropdown menu and select "Create Volume."

d. Configure the volume settings, such as volume type, size, and availability zone.

e. Click on the "Create Volume" button to create the volume from the snapshot.

Step 6. Create the ec2 instance & Attach the Volume to an Instance:

a. Once the volume is created, navigate to the "Volumes" section in the EC2 Dashboard.

b. Find the newly created volume and select it.

c. Click on the "Actions" dropdown menu and select "Attach Volume."

D  the EC2 instance to which you want to attach the volume and specify the device name.

e. Click on the "Attach" button to attach the volume to the instance.

We have created a snapshot of an EBS volume, copied it to another region, created a volume from the snapshot, and attached it to an instance in the destination region.

VIVA QUESTIONS

1. Your database server crashed. Data must be recovered fast. What do you do?
Restore from the latest EBS snapshot or AMI, attach the recovered volume to a new EC2, and bring the service up. Speed comes from having automated backups.

2. EC2 is deleted accidentally — how do you recover data?
Launch a new EC2 and attach EBS volumes restored from snapshots. If no snapshots exist, data is gone.

3. Root volume deleted with EC2. How do you prevent this next time?
Disable “Delete on Termination” for the root EBS volume and enable regular snapshots.

4. Disk is full in production EC2. What will you do?
Extend the EBS volume, grow the filesystem, then clean logs/temp files. Long term: add monitoring and alerts.

5. You want to migrate EC2 to another AWS region
Create an AMI or EBS snapshot, copy it to the target region, then launch a new EC2 there.

6. What happens to EBS when EC2 is stopped?
Nothing — EBS stays intact and keeps all data. Only compute stops.

7. What happens to EBS when EC2 is terminated?
Root volume is deleted by default. Extra attached volumes stay unless explicitly set to delete.

8. Application requires 20,000 IOPS. Which volume will you choose?
Provisioned IOPS SSD (io2 or io1). gp3 may work, but io2 is safer for sustained high IOPS.

9. Can one EBS volume attach to multiple EC2s?
Normally no. Only special “Multi-Attach” volumes support this, and only for specific workloads.

10. You deleted an EBS volume by mistake. How can you get it back?
Restore from snapshot. No snapshot = permanent loss.

11. What happens if a snapshot used by a volume is deleted?
AWS protects dependencies — you can’t delete a snapshot that’s actively in use by a volume.

12. Where are EBS snapshots stored?
In AWS S3 (managed internally by AWS). They’re regional and highly durable.